Practice Assignment 3

Purpose: Calculate entropy changes and use the increase of entropy principle to evaluate process feasibility, irreversibility, and entropy generation in thermodynamic systems.

Due date: Monday, 21SEP2026 11:59PM

Deliverables: Questions 1 and 2 must be completed in MATLAB and submitted as separate .mlx files. Submit this coversheet and any other engineering analysis or supporting work for Questions 3 and 4 as a single .pdf file.

Instructions: Follow the homework format (given, find, schematic, analysis, answer) for full credit on each problem. Numeric answers are provided to check the accuracy of your work. Points are awarded primarily for the engineering analysis component when you complete the problem computations. Clearly document assumptions, property lookups, equations, and process diagrams where requested.

Problems:

- Textbook Problem 8-50: An isentropic steam turbine processes 2 [kg/s] of steam at 3 [MPa], which is exhausted at 50 [kPa] and 100°C. 5 percent of this flow is diverted for feedwater heating at 500 [kPa]. Determine the power produced by this turbine, in [kW].
Ans. W = 2289.5 [kW]

- Textbook Problem 8-116: Refrigerant-134a enters an adiabatic compressor as saturated vapor at 100 [kPa] at a rate of 0.7 [m³/min] and exits at 1 [MPa] pressure. The isentropic efficiency of the compressor is 87%. Determine (a) the temperature of the refrigerant at the exit of the compressor and (b) the power input, in [kW]. Show the process on a T-s diagram with respect to saturation lines.
Ans. (a) T2a = 56.56°C and (b) W = 3.35 [kW]

- Now that you’ve successfully designed a heat engine, you wonder if you can add a turbine to a broom to chase those Sanderson sisters down! You decide to try…steam enters an adiabatic turbine attached to a broom and flows steadily at 45 [m/s], 3 [MPa], and 400°C, and leaves at 75 [m/s], 50 [kPa], and 100°C. The power output from the turbine is 2 [MW].

Find:
(a) List ALL assumptions.
(b) Determine the isentropic power input, in [kW]. (Ans. 3000.5 [kW]) Hint, calculate mass flow rate using known values.
(c) Determine the isentropic efficiency. (Ans. 66.7%)
(d) Determine the rate of entropy generation, in [kW/K]. (Ans. Sgen = 2.817 [kW/K])
(e) Sketch a T-s diagram, complete with labels.

- Given: Harry, Ron, and Hermione are trying to steal food from the Hogwarts kitchen, but the refrigerator stopped working and all of the food spoiled! Hermione, the whiz of all things magical and muggle, knows the problem is the compressor and they can fix it. In your design, steam flows steadily through a compressor at a rate of 0.1 [kg/s]. It enters the compressor as a SLVM at 200 [kPa]. The steam entering the compressor has a quality of 0.983, an enthalpy of 2669.53 [kJ/kg], and an entropy of 7.0335 [kJ/kg-K]. It leaves the compressor at 1200 [kPa] and 50 [m/s].

Find:
(a) List ALL assumptions. You may neglect PE. You may neglect KE at state 1.
(b) Determine the power consumed by the compressor, in [kW]. (Ans. -54 [kW])
(c) Determine the rate of entropy generation, in [kW/K]. (Ans. 0.0264 [kJ/kg-K])
(e) Sketch a T-s diagram, complete with labels.

Schematic: